Multi-optimization formulation

Objective function

The storage system is optimized to reduce renewable surplus in the grid. Meanwhile, it is also configured to discharge energy during periods characterized by high carbon intensity in electricity production. The objective function is show in Eq. [1].

| mini=0Te10∙εc+εh+Suel,i-yi,TCM_h+yi,PCM_h+yi,TCM_c+yi,PCM_c-ω∙xi,TCM_h+xi,PCM_h+xi,TCM_c+xi,PCM_c∙βi | [1] |
| --- | --- |

Where yi,TCM_h, yi,TCM_c,yi,PCM_h,yi,PCM_c represent the electrical energy that is used for charging TCM_h, TCM_c, PCM_h and PCM_c at time instance i, respectively. Likewise,  xi,TCM_h,  xi,TCM_c, xi,PCM_h, xi,PCM_c denote the electrical energy that is used for discharging; Suel,i is the electricity renewable surplus at time instance; βi is the carbon content in ton/GWh; ω is a hyperparameter weight for dual objectives. Furthermore, two additional variables are introduced, which will be discussed later. The simulation time step is 1 hour which is identical to the historical data retrieved from ENTSOE platform, and the simulation horizon is 12 hours.

Constraints

In this study, the operational SoC is constrained between 20% and 75% for PCM storages, while 15% to 80% for TCM storages for any time instance i. The SoC update rules are defined according to Eqs. [2-5].

| SoCi, PCM_c=SoCi,PCM_c+100⋅yi,PCM_c⋅EERi-xi,PCM_c⋅EERiCmPCM_c⋅ηPCM-Fi, c | [2] |
| --- | --- |
| SoCi,PCM_h=SoCi,PCM_h+100⋅yi,PCM_h⋅COPi-xi,PCM_h⋅COPiCmPCM_h⋅ηPCM-Fi, h | [3] |
| SoCi,TCM_c=SoCi,TCM_c+100⋅yi,TCM_c⋅α⋅ηTCM, ch-xi,TCM_c⋅α⋅ηTCM, discCmPCM_c | [4] |
| SoCi,TCM_h=SoCi,TCM_h+100⋅yi,TCM_h⋅α⋅ηTCM, ch-xi,TCM_h⋅α⋅ηTCM, discCmPCM_h | [5] |

Equations [2, 3] indicate the SoC update rules for PCM storages, where yi and xi represent the electrical energy that is used for charging and discharging. In order to correlate the electrical energy with the absolute thermal energy capacity CmPCM_c (MWh), the hourly COP and EER are calculated to convert electrical energy to thermal energy. The detailed calculation can be referred to [Add calculation detail]. Moreover, a coefficient, ηPCM, is introduced to represent the efficiency of PCM storage systems during both charging and discharging phases. The SoC update rules for TCM storages are presented in Equations [4, 5]. Different from PCM storage systems, a standard conversion factor, α, is used to convert electrical energy to thermal energy, instead of COP or EER. The efficiencies for charging and discharging are represented by ηTCM,ch and ηTCM,disc, respectively. Additionally, the dynamic energy losses of PCM storage are denoted by Fi,  c and Fi,  h. Such dynamic losses are modelled as a function against SoC, as shown in Eq. [6].

| Fi=floss∙1-e-k∙SoCt-SoCminSoCmax-SoCmin | [6] |
| --- | --- |

Where, floss,  c denotes the constant energy loss, with which the storage is expected to drop from SoCmax to SoCmin in 24 hours as shown below (Eq. [7]). Moreover, SoCt is the SoC of storage at time instance t, and k is a hyperparameter. In this study, k is selected as 7. It is worth noting that the exponential term provides a decay with respect to the current SoC. That is, the fraction of the constant energy loss increases with higher SoC levels and decreases with lower SoC levels, thereby capturing the intuitive notion that energy loss is driven by the temperature difference.

| floss=SoCmax-SoCmin24 | [7] |
| --- | --- |

However, due to the non-convex nature of this function, the dynamic energy loss cannot be directly integrated into optimization framework. Hence, auxiliary variables ε_h and ε_c are introduced to relax the non-convex term. Rather than enforcing equality directly in the SoC update rule, a convex epigraph constraint is imposed. In particular, it should be noted that the set SoCt, ε_h:ε_h≥e-k∙SoCt-SoCminSoCmax-SoCmin  SoCt, ε_c:ε_c≥e-k∙SoCt-SoCminSoCmax-SoCminis convex, therefore, the loss is reformulated as:

| Fi=floss∙1-ε | [8] |
| --- | --- |

To enforce ε to be the minimum value, a penalty term (e10∙ε) is added to the original objective function.

Furthermore, to minimize energy waste caused by the constant thermal losses in PCM storage, an additional constraint is introduced, shown in Eq. [9]. This constraint ensures that surplus energy is not used to charge PCM storage if there is no anticipated space heating or cooling demand (Dh&c) within the next 12 hours (i+12), based on the cumulative future demand forecast.

| FDi,h&c=τ=ii+12Dτ,h&c | [9] |
| --- | --- |

The constraints regarding charging and discharging actions are formulated in Equation [10 to 17].

| yi,PCM_h≤mini(FDi,h, Suel,i)∙ui,PCM_h | [10] |
| --- | --- |
| yi,PCM_c≤mini(FDi,c, Suel,i-yi,PCM_c)∙ui,PCM_c | [11] |
| xi,PCM_c≤Di,c∙(1-ui,PCM_c) | [12] |
| xi,PCM_h≤Di,h∙(1-ui,PCM_h) | [13] |
| yi,TCM_h≤mini(FDi,h-xi,TCM_h, Suel,i-yi,PCM_c-yi,PCM_h)∙ui,TCM_h | [14] |
| yi,TCM_c≤mini(FDi,c-xi,TCM_c, Suel,i-yi,PCM_c-yi,PCM_h-yi,TCM_h)∙ui,TCM_c | [15] |
| xi,TCM_c≤(Di,c-xi,PCM_c)∙(1-ui,TCM_c) | [16] |
| xi,TCM_h≤(Di,h-xi,PCM_h)∙(1-ui,TCM_h) | [17] |